CommandCenter System Settings Guide

Audience: customer on-site maintenance / commissioning engineers    Date: 2026-08-19    Software version: V2.15.10

How to read this guide: This guide explains how to use and configure the "System Settings" functions only; it does not cover PLC protocol, registers, camera commands or other development-level topics. Operator daily-use instructions are in the separate User Manual (Operator Handbook).

Save = apply now: All changes take effect immediately after clicking [Save] in the System Settings window; no restart is required (devices reconnect automatically after a short drop; see the log message "Configuration saved and applied hot").

1. System Overview and Main Screen

1.1 System Components

| Device | Brand / Model | Role in this system |
| --- | --- | --- |
| Host PC | The machine running this software (IPC) | Runs CommandCenter: central display, image archiving, result reporting |
| PLC | Inovance | Production-line master controller; issues scan/camera requests and reads results |
| Smart cameras | Keyence IV4-500CA (x2) | Shoot per point, judge OK/NG, push images to the host PC via FTP |
| Scanner | Keyence SR series | Reads product serial number (SN) |

1.2 Main Screen Layout

The main screen consists of three parts: Title Bar + Window Matrix + Bottom Status Bar.

┌──────────────────────────────────────────────────────────────┐

│ Model:[xxx v] Serial:[read-only][Manual] Total:0 OK:0 NG:0 [Settings] ●PLC●Scanner●Cam1●Cam2│

├──────────────────────────────────────────────────────────────┤

│  ┌──────┐ ┌──────┐ ┌──────┐ ┌──────┐ ┌──────┐                │

│  │  W1  │ │  W2  │ │  W3  │ │  W4  │ │  W5  │ <- Window grid │

│  └──────┘ └──────┘ └──────┘ └──────┘ └──────┘                │

├──────────────────────────────────────────────────────────────┤

│ Status: Waiting for PLC master...                            │

└──────────────────────────────────────────────────────────────┘

Title bar: model drop-down (switch model directly) -> serial number -> Total / OK / NG counters -> Settings button -> connection indicator lights (green = connected, red = disconnected/fault; PLC light has three states: green = master connected, yellow = listener ready waiting for master, red = listen failed; hover for tips) -> language toggle button.

Window matrix: each cell shows one camera feed; after each inspection the latest image refreshes in rotation. Each window shows an OK/NG badge at its bottom-right (green = OK, red = NG). When a new scan request (new part) arrives, all windows and badges are cleared and reset automatically.

Bottom status bar: shows the current process stage (waiting for scan / waiting for camera / shooting / model switch complete, etc.).

Double-click any window to view it full-screen; double-click again or press Esc to restore.

1.3 Common Operations on the Main Screen

Switch product model: select the target model from the "Model" drop-down in the title bar; it takes effect immediately (devices stay connected, no restart). Model candidates can be added/removed under System Settings -> PLC area -> Product Model Config...

Manual serial input: when the scanner cannot read the code or the SN needs changing, click [Manual] next to the serial box and type the serial number in the dialog (Enter = OK, Esc = Cancel; an empty input is not accepted).

Scanner read-failure prompt: when the scanner fails to read, the program pops a reminder. Click [Manual Input] to record this part now, or [Later] to do it later (this part is NOT released; the PLC waits until it is recorded). You may check "Don't remind me today" in the dialog.

2. Accounts, Login and Security

| Account | Default password | Opens | Permissions |
| --- | --- | --- | --- |
| admin | admin123 | System Settings window | Change configuration, save and apply hot |
| dev | dev123 | Developer Mode window | Manual communication tests + account management (reset any account's password) |

Clicking the [System Settings] button opens a login dialog first; too many wrong passwords will temporarily block login.

The login dialog has a built-in "Change Password" panel (admin only): verify the current password -> new password entered twice and at least 6 characters -> Save.

"Remember password": when checked, the account and password are stored encrypted (bound to the current Windows user) and auto-filled next time; it stops working after switching Windows users or computers. Admin and developer records are mutually exclusive: logging in with one account clears the remembered record of the other.

The admin password is stored only as a SHA-256 hash. Change it via the "Change Password" panel in the login dialog; never edit the configuration file manually.

Logging in with the developer account (dev) opens the Developer Mode window, where the "Account Management" area at the top can reset any account's password (useful when the on-site admin forgets it).

3. Entering System Settings

Click the [System Settings] button on the right side of the title bar.

Enter the admin account (admin) and password (default admin123) in the login dialog.

On success the System Settings window opens. Configure the sections below, then click [Save] to apply immediately.

The System Settings window has six configuration areas: PLC, Camera, Scanner, Display, Image, Security, plus auxiliary buttons (Product Model Config, Window/Point Config, Directory Structure Config, etc.). Each area is described below.

4. PLC Settings Area

4.1 Communication Parameters

| Item | Default | Description |
| --- | --- | --- |
| Bind IP | 0.0.0.0 (all adapters) | Listen address of the host acting as a Modbus TCP slave. If the IPC has a fixed IP, you may enter it here |
| Port | 502 | Standard Modbus TCP port through which the PLC master connects |
| Slave Unit ID | 1 | Slave station number; must match the PLC master configuration |

4.2 Product Model Config... (button)

Opens the "Product Model Config" dialog, which maintains the "model <-> PLC index" mapping (default: Z121=1, U171=2):

Add: click [Add] at the top-right of the table to insert a row, then enter the model name (the index can also be edited).

Delete: tick the rows to delete (leftmost check column) and click [Delete Selected]; the Delete key does the same.

OK: after validation the mapping is written back; it is persisted only after clicking [Save] in System Settings. Cancel closes without saving.

Models added here automatically appear in the model drop-down on the title bar and in Window/Point Config, ready for use.

Note: The "Product Model" entries here are only used to send the model index/name to the PLC (protocol-level coordination). For day-to-day model switching, use the model drop-down on the title bar of the main screen; no need to enter System Settings.

5. Camera Settings Area

Each row in the camera table represents one camera; the number of rows equals the number of cameras (add/delete rows to add/delete cameras). The table is displayed sorted by Camera ID but saved in the original configuration order, so you do not need to care about order.

| Column | Meaning | On-site defaults / notes |
| --- | --- | --- |
| Camera ID | Real Keyence camera number, consistent with the archive directory number | Top camera = 2, bottom camera = 1 (do not change numbering by swapping row order) |
| Name | Name shown in UI / logs | Top camera, bottom camera |
| IP address | Camera IP | Top 19.87.6.213, bottom 19.87.6.212 |
| Command port | Raw protocol communication port | 8500 (per actual camera Navigator settings) |
| FTP upload dir | Directory the camera pushes images to; the host monitors it for images | Top D:\IV存图\2, bottom D:\IV存图\1 (paired opposite to the physical positions) |
| Image source | How images are obtained | Ftp only (Tcp removed) |
| Trigger / trigger+read command | Shooting command | T1 (trigger only) / T2 (trigger + read judgment, default path) |
| Output format | Camera judgment response format | Must be exactly 2 digits: 00 standard / 01 detailed (an invalid value makes that camera fail to trigger) |
| Response timeout | Timeout waiting for camera response | 5000 ms; increase if the camera is slow |
| PLC request/result index | Channel addresses between camera and PLC (DataStore index) | Top = request 2 / result 5, bottom = request 3 / result 6; 0 = channel not configured (not polled) |

Common fault: If a camera fails to trigger directly and the log reports "Failed to set judgment output format", the "Output Format" column most likely holds an invalid value (it must be exactly 2 digits, e.g. 00).

Note: Camera IP, FTP upload dir and camera ID are all configured in this table. The host runs NO FTP server; just make sure the camera-side push target directory matches "FTP Upload Dir" in this table.

6. Scanner Settings Area

Scanners are split into a "TCP table" and a "Serial table"; each row is one scanner. A scanner is only connected when its "Enabled" box is ticked.

6.1 TCP Table (Ethernet scanner)

| Column | Meaning | On-site defaults |
| --- | --- | --- |
| Enabled | Ticks to connect this scanner | Checked |
| IP address | Scanner IP | 19.87.6.100 |
| Port | Scanner listening port | 9004 |
| Trigger command | Command sent automatically after connecting (\r\n appended automatically) | LON (turns the laser on to start reading); leave empty to not send |
| Ignore texts | Filter list for read-failure/error texts; comma-separated, case-insensitive; * at the end = prefix match | ERROR,ERR,NG,NOREAD |

6.2 Serial Table (RS-232 scanner)

| Column | Meaning | On-site defaults |
| --- | --- | --- |
| Enabled | Ticks to connect this scanner | Unchecked (TCP scanner used by default) |
| Port name | Serial port | COM3 |
| Baud rate | Serial baud rate | 115200 |
| Stop bits | Stop bits | 1 |
| Parity | Parity | None |

In TCP mode the scanner does NOT send data just because it is connected; a trigger command (default LON) must be sent first to enter read mode. The program sends it automatically after each (re)connect.

The scanner accepts only ONE TCP client: if tools such as a network debug assistant hold the connection, the host cannot connect and shows "repeated disconnection". Close the debug assistant connection before testing the program.

Read-failure texts (e.g. ERROR) are filtered by the "Ignore Texts" list, so they are not shown as a serial number or archived, and the PLC is notified immediately. If the on-site scanner outputs another error text, just add it to the list.

Note: If the scanners section in the configuration is empty, the program automatically adds a default TCP scanner (19.87.6.100:9004 / LON / enabled). Configure scanners here in the Settings window; editing the legacy single-object "scan" section in old JSON does NOT work.

7. Display Settings Area

7.1 Window Matrix: Rows x Columns + Auto Fit

Check "Auto Fit": rows and columns are computed automatically from the current product model and each camera's point table - at most 7 columns, missing cells concentrated in the last row, windows as large as possible. The row/column input boxes are greyed out while Auto Fit is on.

Uncheck "Auto Fit": the manually entered rows x columns IS the total window count (what you see is what you get). If there are fewer points than cells, the extra cells become "empty windows" (no point): they still fill the display area but show no image.

When the number of rows is <= 10, windows always fill the display height equally; if rows exceed 10, a vertical scroll bar appears on the right of the window area.

The window matrix re-arranges automatically when the product model changes.

7.2 Display Switches

| Switch | Description |
| --- | --- |
| Show window index | Shows the window number at the top-left of each cell (default on) |
| Hover tip | Shows a "double-click to zoom / double-click to restore" bubble on hover (default on) |
| Window OK/NG badge | Shows the OK/NG badge at the bottom-right of each window (default on) |
| Title-bar field toggles | Each title-bar field (model/serial/counters, etc.) can be shown or hidden |
| OK/NG color names | On-site convention: OK = green, NG = red; color names are configurable here |

7.3 Window/Point Config... (button)

Opens the "Window/Point Config" dialog, which has two areas:

Upper area: Window <-> Point matrix

Each cell represents one display window and shows "camera - point number". The default layout is top camera first, then bottom camera. Operations:

[Edit Point]: pick a point from ALL points of the current model's camera point tables (choosing a point already used by another window automatically swaps it with that window to keep uniqueness).

[Swap Position]: swap points of any two windows (including across cameras). Click the first window (highlighted in sky blue), then the second to finish; the two swapped windows flash green as confirmation.

[Restore Default]: reset the factory layout for this model and re-enable all windows.

[Disable/Enable]: right-click a window cell to stop/restart shooting a point (disabled cells show a grey "disabled" background); the disabled state follows the point when it moves.

Empty windows (no point) support swapping but do not support editing points or disable/enable.

Lower area: Camera Program Mapping (point -> program number)

The on-site windows/points are shot by two cameras in a shared arrangement; each point corresponds to one program in its camera. First choose a camera in the camera drop-down, then choose the product model, then edit the "Point / Program number" table:

The point column = window points (count = number of windows); the program column = "No switch" + 0~127 (program count and numbers depend on the programs actually installed in the camera, unrelated to the window count).

"No switch" = keep the camera's current program. If a point is not shot by this camera, just delete that row.

The mapping is stored per camera + model. Before shooting, the program automatically sends a PW command to switch to the corresponding program first.

A newly added camera automatically gets its own empty mapping table; saved mappings are not lost.

8. Image Settings Area

8.1 Basic Items

| Item | Description |
| --- | --- |
| Save root directory | Root directory where inspection images are archived |
| Configure directory structure... | Opens the directory structure dialog (see 8.2) |
| FTP listen directory | Global fallback directory for camera image arrival (each camera is usually configured separately in the camera table) |

8.2 Configure Directory Structure... (button)

Configures the archive directory levels, file name rule and cleanup policy:

Default structure: root / {年月日} / {SN} / {相机} / OK|NG / image (the {相机} level separates top and bottom camera images so same-numbered points do not overwrite each other).

Each level must be exactly ONE directory name or rule (e.g. {年月日}, {SN}, {相机}, OK, NG). Add levels one by one; do NOT paste an entire path as one level (this creates deeply nested directories; the program repairs already-broken configurations automatically).

Timestamp suffix switch: when checked, archive file names get a _timestamp suffix (default on), preventing repeated shots of the same point from overwriting history.

Image keep days: keep only the last N days (default 30; 0 = no auto cleanup). A background task deletes expired date directories daily (it only cleans under the save root directory and never touches the camera FTP directories).

9. Security Settings Area

Admin login switch: when off, the [System Settings] button no longer requires login; when on, every entry into System Settings requires the admin account and password.

Account/password maintenance is done in the login dialog (change password, remember password). The System Settings window itself has no account-management area.

10. Interface Language Switch

Entry: the language toggle button next to the [System Settings] button in the title bar (shows "English" on the Chinese UI and "中文" on the English UI). Click to switch and save instantly; it persists after restart.

Only UI text switches; logs stay in Chinese for remote troubleshooting. Terms such as OK/NG, PLC, IP, SN are not translated.

11. On-site Parameter Quick Reference

| Device | Brand / Model | IP address | Port | Key parameters |
| --- | --- | --- | --- | --- |
| Host PC (IPC) | - | 19.87.6.230 | 502 (listen) | Runs CommandCenter; Modbus slave listening on 502 |
| PLC | Inovance | 19.87.6.1 | - | Modbus master, connects to the host on 502 |
| Camera 1 (top) | Keyence IV4 | 19.87.6.213 | 8500 (cmd) | FTP push to D:\IV存图\2 |
| Camera 2 (bottom) | Keyence IV4 | 19.87.6.212 | 8500 (cmd) | FTP push to D:\IV存图\1 |
| Scanner | Keyence SR | 19.87.6.100 | 9004 | Trigger command LON |

All devices must be on the same subnet as the host (19.87.6.x / 24) and able to ping each other.

The top/bottom cameras' FTP directories are paired OPPOSITE to their physical positions (top camera pushes to \2, bottom camera to \1). Keep this convention when changing directories.

12. Troubleshooting (On-site Quick Reference)

| Symptom | Possible cause | Action |
| --- | --- | --- |
| PLC light red | Port 502 occupied / wrong bind IP / firewall blocking | Hover the light for hints; check port usage and firewall inbound 502 |
| PLC light yellow | Slave ready but master not connected | Check that the PLC master program is running and points to this PC:502 |
| Scanner keeps disconnecting | A debug assistant or other tool holds the connection (scanner accepts one client) | Close the debug assistant connection and restart the program |
| FTP image never arrives | Camera push target differs from the program's listen directory / camera has not pushed yet | Verify the camera push directory matches "FTP Upload Dir" in the camera table (top -> \2, bottom -> \1) |
| A camera fails to trigger, log reports "Failed to set judgment output format" | "Output Format" holds an invalid value | Change it to exactly 2 digits (00/01/02/03) |
| UI buttons respond very slowly | Network operation blocking the UI thread (should not happen) | Restart the program and report the log to maintenance |
| Window zoom not responding | Double-click landed outside the image area | Double-click on the image area (the whole window) |