Host Computer (PC) Software Operation Manual

Audience: Field operators and team leaders.
What this covers: Power-on, reading the interface, daily operation, and what to do when something goes wrong. It does not explain internal principles.
For engineers: The companion technical document is docs/CommandCenter.md (communication protocol and configuration details).

1. Power-On and Startup

1. Turn on the computer and wait for Windows to finish starting.

2. Double-click the CommandCenter (host inspection program) icon on the desktop.

3. Wait 2~3 seconds. When the main window fills the screen and rows of inspection windows appear, the program has started successfully.

4. Normal production does not require login. Only maintenance personnel need to enter an administrator account when changing parameters by clicking "System Settings" in the top-right corner.

If startup fails: check the bottom-right corner of the screen for error messages; make sure the Config folder has not been modified. If it still won't start, call an engineer. Do not delete files yourself.

2. Getting to Know the Main Interface

┌──────────────────────────────────────────────────────────────────┐

│ Model:[U171 ▾]  Serial No.:[SN12345][Manual Entry] Total:0 OK:0 NG:0 [System Settings]  ●PLC ●Scanner ●Cam1 ●Cam2│

├──────────────────────────────────────────────────────────────────┤

│  ┌──────┐ ┌──────┐ ┌──────┐ ┌──────┐ ┌──────┐                    │

│  │  W1  │ │  W2  │ │  W3  │ │  W4  │ │  W5  │   ← Inspection window matrix │

│  │image │ │image │ │image │ │image │ │image │     one cell = one inspection point │

│  └──────┘ └──────┘ └──────┘ └──────┘ └──────┘                    │

├──────────────────────────────────────────────────────────────────┤

│ Status: Waiting for inspection…                                  │

└──────────────────────────────────────────────────────────────────┘

Explanations:

| Location | Name | What it is / How to read it |
| --- | --- | --- |
| Top-left | Model | The product model currently being produced. Click the dropdown to switch directly. After switching, the cameras automatically switch to the matching program; no restart needed |
| Center | Serial No. | The barcode of the current workpiece. It fills in automatically when the scanner reads it (read-only display) |
| Button | 【Manual Entry】 | Used to manually enter or modify a barcode when the scanner fails to read it |
| Title bar | Total / OK / NG | Inspection point count for the current single workpiece: OK = green, NG = red. It resets automatically to zero each time a new workpiece arrives |
| Top-right | ● Dot lights | Four lights: PLC, scanner, camera 1, camera 2. Green = normal, red = problem (PLC yellow = waiting for the master station, which is normal) |
| Center | Window matrix | Each cell is the camera image of one inspection point. The latest image refreshes automatically after each shot; a green border in the bottom-right of a cell = this point is OK, red border = NG |
| Bottom | Status bar | Shows what the system is doing right now: waiting for a scan, taking a photo, switching model, etc. |

Reminder: Every time a new workpiece arrives, all windows are cleared to dark gray and the counter resets to zero. This is normal and means the current workpiece has started inspection from scratch.

3. Daily Standard Procedure

1. Power on and start the program.

2. Confirm the four dot lights in the top-right corner: PLC, scanner, camera 1, camera 2 must all be green (yellow PLC can wait). If any light is red, do not start work; see Section 8.

3. Confirm the Model in the top-left corner is the model for today. If not, click the dropdown to switch, and wait until the status bar shows "Model switch complete".

4. Place the workpiece → the scanner reads the barcode automatically → the cameras take photos automatically → each window shows its image → each point is automatically judged OK/NG.

5. During normal production no operation is needed; just watch the window images and counters.

4. Serial Number (SN) — Very Important

• Normal: When the scanner reads a barcode, it appears automatically after "Serial No.:". Nothing to do.

• Scanner failed to read, or read the wrong code: Click the 【Manual Entry】 button next to it → an input box pops up → enter or modify the barcode → press Enter to confirm (press Esc to cancel).

• When entering, note: you cannot confirm an empty value; if you don't actually want that code, click Cancel.

⚠️ The serial number determines which folder photos are archived to (Date/Barcode/...), so if the barcode is wrong, the photos are stored in the wrong place. Each time you change workpieces, check whether the serial number matches the workpiece.

5. When the Scanner Has a Problem (Key Section)

Symptom

A popup appears: "Scanner error. Please check the scanner or perform manual entry."

Cause

The scanner failed to read a code this time (dirty barcode / too far away / loose cable / scanner broken).

What to do (in order)

1. Look at the scanner: Is the laser flashing? Is the cable loose? Straighten the barcode and bring it closer, then try again.

2. Once the scanner recovers: the popup closes automatically. Nothing to do.

3. If it can't be fixed right away: Click 【Manual Entry】, manually type in this workpiece's barcode, and production continues.

4. Clicking 【Later】: only means "don't bother me for now". This workpiece still owes a barcode, and the program will keep waiting — you must fill it in later via 【Manual Entry】, otherwise the process will be stuck.

5. The popup has a ☐ Don't remind me today checkbox: when checked, no more popups today (the workpiece still needs manual entry; it just stops nagging you). Suitable for cases like "the scanner is being repaired, use manual entry to get through today".

⚠️ The popup is not nothing happening — it means this workpiece's code was not read. Don't keep clicking 【Later】 as if it didn't happen — if you don't do the manual entry, this workpiece cannot pass.

6. Zooming In on a Cell's Image

• Double-click any window → it enlarges to full screen for a closer look (the image keeps refreshing in real time while enlarged).

• Double-click again / press Esc → restores to its original position.

• While in full screen, to look at another cell: simply double-click that other window to switch.

7. Where Photos Are Stored (Finding Images/Records)

• The program automatically saves each inspection photo to the computer's image directory. The structure is:

E:\Images (image root directory)

└─ 2026年08月17日 (date)

└─ SN12345 (barcode number)

└─ Upper camera (which camera took it)

├─ OK (this point passed inspection)

└─ NG (this point failed inspection)

└─ 001_20260817_164022_461.jpeg (photo)

• To look up the photos of a specific workpiece: ask the team leader/engineer to open the corresponding folder.

• Photos are kept for 30 days by default and then auto-cleaned. Copy important ones out early.

8. What to Do When a Connection Light Is Red (On-Site Quick Reference)

| Symptom | What it means | What the operator should do |
| --- | --- | --- |
| PLC light red | PLC communication is down | Don't start work; call an engineer |
| PLC light yellow | Waiting for the PLC master station to connect | Normal waiting; nothing to do |
| Scanner light red | Scanner not connected | Check the scanner's cable/power; hold it up with 【Manual Entry】 per Section 5 |
| Camera light red | Camera communication is down | Call an engineer (check the camera network cable / image-push configuration) |
| Window never shows an image | The camera took the shot but the image didn't arrive | Call an engineer to look at it; don't keep triggering repeatedly |

9. Prohibited Operations (Red Lines — Must Be Observed)

1. Don't click "System Settings" — it requires an administrator password and is for maintenance personnel to change parameters. Operators should not enter it.

2. Don't touch other windows on the computer, don't turn off the firewall, don't unplug network cables.

3. Don't manually delete/modify files in the image folders.

4. When a popup or anomaly appears, follow the prompt first; if you can't resolve it, call an engineer; don't restart the computer directly (it will lose the current workpiece's data).

5. Don't change the product model on your own; model changes should follow the team leader's arrangement.

10. Shift Handover / End of Day

• The program can stay running; it doesn't need to be closed.

• When you need to shut down, just click Close in the top-right corner normally.

• If there were any issues that day such as scanner errors or camera errors, tell the next shift/engineer verbally or write it in the handover log, to make troubleshooting easier.

Frequently Asked Questions — Quick Answers

• Q: Why are all the windows dark gray?

A: A new workpiece has started and the windows were just cleared. The image will appear automatically once the camera finishes shooting. If no image ever appears, follow Section 8.

• Q: Why does the counter stay frozen?

A: Check whether the status bar is stuck at some step; if there is a popup, handle it first; if it still won't move, call an engineer.

• Q: What if I selected the wrong model?

A: Click the model dropdown in the top-left corner, select the correct model, and wait for the status bar to show "Model switch complete". No restart needed.

• Q: Why does production still not move after I dismissed a popup?

A: Most likely this workpiece's barcode has not been entered yet (see Section 5, item 4). Click 【Manual Entry】 to fill in the code and it will be fine.