System Overview and Device List

1 System Composition and Communication Overview

The host PC acts as the Command Center, running in the rhythm of "scan to get the SN → cameras take photos → save images → display in the corresponding point windows → report the result to the

PLC → move to the next station". Communication overview:

| Device | Manufacturer/Model | Communication Method | Protocol |
| --- | --- | --- | --- |
| Smart Camera | Keyence IV4-500CA | TCP/IP no-protocol communication (×2 connections) | ASCII commands + FTP image upload (only FTP image acquisition is retained since V1.12.18; Tcp/BR direct read retired) |
| PLC | Inovance | Modbus TCP (on site the PLC is the master; the host PC acts as a slave listening on 502, reversed since V1.12.11) | Modbus holding registers |
| Scanner | Keyence SR series | Serial RS-232 / Ethernet TCP/IP no-protocol (select one since V1.8.0) | Barcode text line |

2 Network Topology

┌──────────────┐
                         │   Host PC    │
                         │ 19.87.6.230  │
                         └───────┬──────┘
                                │ LAN (19.87.6.x / 24)
        ┌──────────┬─────────────┼──────────────┬─────────────┐
        │          │             │              │             │
   ┌────┴───┐ ┌────┴────┐  ┌─────┴─────┐  ┌─────┴─────┐ ┌─────┴─────┐
   │ PLC    │ │ Cam 1   │  │ Camera 2  │  │  Scanner  │ │(Reserved) │
   │Inovance│ │ Upper   │  │ Lower     │  │ KeyenceSR │ │           │
   │ .1     │ │ .213    │  │ .212      │  │ .100:9004 │ │           │
   └────────┘ └─────────┘  └───────────┘  └───────────┘ └───────────┘

The host PC's own IP `19.87.6.230` does not need to be written into the program configuration (it is the NIC address of the running host, configured in the system network).

All devices must be on the same subnet as the host PC (`19.87.6.x / 24`) and be able to ping each other.

3 Device List Quick Reference

| Device | Brand/Model | IP Address | Port | Key Parameters |
| --- | --- | --- | --- | --- |
| Host PC (the machine running this program) | - | `19.87.6.230` | `502` (listening) | Runs the CommandCenter host PC program; acts as a Modbus slave listening on 502, waiting for the PLC master to connect and read/write the host PC's register area (V2.7 protocol `40001~40011`, three-step handshake) |
| PLC | Inovance PLC (master) | `19.87.6.1` | - | Acts as the Modbus master, connects via TCP to the host PC `19.87.6.230:502` to read/write the host PC's holding register area (protocol details in Part 5) |
| Camera 1 | Keyence IV4 (upper camera) | `19.87.6.213` | 8500 (command port) | T1/T2/RT/PW/OF commands; FTP uploads images to `D:\IV存图\2` (jpeg+iv4p dual files, the only image acquisition method since V1.12.18; finalized in V1.12.22, directory pairing fixed in V2.13.3 — the upper camera pushes to `\2`, opposite to its mounting position) |
| Camera 2 | Keyence IV4 (lower camera) | `19.87.6.212` | 8500 (command port) | Same as above; FTP uploads images to `D:\IV存图\1` (the lower camera pushes to `\1`, opposite to its mounting position); the two cameras are configured independently |
| Scanner | Keyence SR series | `19.87.6.100` | `9004` | Trigger command `LON` (details in Part 3 and Part 7) |

Note: The PLC is the on-site control master; port 502 is the standard Modbus TCP port. Camera command port 8500 and scanner port 9004 are

program defaults. If the actual device settings differ on site, the device side prevails (they can be changed in the settings form).